ESP32 Fingerprint Attendance System

Technical Documentation

Multi-tenant biometric attendance & retail platform

Version 3.0  |  September 2026

Firmware 1.10.0  ·  Edge functions (Deno)  ·  Next.js dashboard (PWA)

# Contents

# 1. Project Overview

The ESP32 Fingerprint Attendance System is a multi-tenant biometric attendance and retail platform. ESP32 devices with an R503 capacitive fingerprint sensor record attendance at the unit level (a classroom, an office section, a salon). A Next.js dashboard backed by a single cloud Supabase project provides reporting, device provisioning, remote fingerprint enrollment, institution configuration, and — for shop-type tenants — a point-of-sale and loyalty module. One cloud deployment serves every institution, with each tenant's data isolated from the rest.

The system replaces manual paper roll-calls with a tamper-resistant fingerprint scan. Records are timestamped in the institution's own timezone; absent marks are applied automatically at end of day by a scheduled job. Devices keep working through WiFi outages by queueing scans to on-board flash.

## 1.1 Key capabilities

- Biometric identification — R503 capacitive sensor, up to 127 enrolled fingerprint slots per device, on-sensor matching (~0.3 s).

- Multi-tenant — one cloud deployment serves any number of institutions (school / office / shop) with strict per-tenant data isolation.

- Per-member-type scan modes — present/absent (one scan marks the day) or time-in / time-out (paired scans), configurable separately for students and staff.

- Punctuality — optional per-institution late-arrival and early-departure thresholds stamp each scan on_time / late / early_leave.

- Tracked weekdays — an explicit Mon–Sun set; scans on untracked days are ignored and generate no absences (replaces the old skip-weekends flag).

- Offline resilience — every scan is written to SPIFFS flash before any network attempt; a rotate-then-process queue survives reboots and power loss.

- Automatic absent marking — pg_cron invokes the mark-absent edge function daily; it respects holidays, tracked weekdays, and academic periods.

- On-device OLED — optional SSD1306 128x64 screen shows a live status card (idle clock, scan verdict, enrollment step, portal, OTA, degraded/offline); the firmware runs headless if it is absent.

- Remote fingerprint enrollment — admins queue jobs from the dashboard; devices pick them up, capture the finger, and report back. Stuck jobs are re-delivered and self-healed.

- OTA firmware updates — the device checks GitHub Releases on boot (after a random jitter) and self-flashes a strictly newer build.

- Academic calendar — terms/periods plus recurring and one-off holidays.

- Retail / loyalty (shop tenants) — clients, catalog (products & services), point of sale, and a punch-card loyalty module with a redemption loop.

- Role-based access — six roles (platform_admin, super_admin, admin, teacher, staff, cashier) with institution- and device-scoped visibility.

- Installable PWA — the dashboard installs to a home screen / desktop via a web manifest and a minimal service worker (no offline cache).

# 2. System Architecture

Three tiers: ESP32 firmware, the Supabase cloud backend (PostgreSQL + Edge Functions + pg_cron), and the Next.js dashboard on Vercel. The ESP32 talks only to the edge functions over HTTPS; the dashboard talks only to PostgreSQL, server-side, through the service role.

## 2.1 Component overview

| Tier | Technology | Role |
| --- | --- | --- |
| ESP32 firmware | Arduino / FreeRTOS (single sketch) | Scan fingerprints, queue attendance to flash, drive the OLED, provision and self-update over HTTPS |
| R503 sensor | UART, capacitive | Fingerprint capture, template storage, and matching — entirely on-device |
| DS3231 RTC | I2C | Wall-clock time across reboots and outages; re-synced from NTP on boot and reconnect |
| SSD1306 OLED | I2C (optional) | On-device status display; shares the I2C bus with the RTC |
| Supabase Edge Functions | Deno / TypeScript | HTTPS API for device provisioning, attendance logging, enrollment, and the daily absent job |
| Supabase PostgreSQL | PostgreSQL 17 + pg_cron + pg_net | Multi-tenant data store; schedules and drives daily absent marking |
| Next.js dashboard | App Router + shadcn/ui + TailwindCSS | Admin web interface; all Supabase access is server-side via the service role |
| Vercel | Hosting | Dashboard deployment (production tracks the oled-integration branch) |

## 2.2 Data flow

- A finger is scanned. The R503 matches it to a stored template and returns a slot number (FID) and a confidence score.

- The firmware resolves the FID to a member via the on-device fid_map.csv, writes the scan to /queue.txt in SPIFFS, and signals the network task.

- The network task rotates /queue.txt to /queue_inflight.txt and POSTs each record to /log-attendance over HTTPS with the device's x-device-secret header and a body of { device_id, sid, scan_type, scan_id, ts, ... }.

- /log-attendance authenticates the device, resolves the member within the device's institution, applies the institution's tracked-weekday / scan-mode / punctuality rules, deduplicates, and inserts an attendance row.

- A trigger bumps institution_activity.last_change_at. The dashboard polls /api/changes every 12 s and calls router.refresh() only when that watermark advances.

- pg_cron runs mark-absent daily. It calls net.http_post() (pg_net) to invoke the edge function, which inserts absent rows for every active member with no present scan that day, per institution and timezone.

# 3. Hardware

A device is built from off-the-shelf modules on a custom passive carrier PCB, housed in a 3D-printed enclosure. Unit cost is roughly GHS 1,000.

## 3.1 Bill of materials

| Item | Part / spec | Notes |
| --- | --- | --- |
| Microcontroller | ESP32 dev module (dual-core Xtensa LX6, 240 MHz, 4 MB flash, WiFi) | Seats in the two 15-pin headers on the carrier PCB |
| Fingerprint sensor | R503 capacitive (UART, up to 127 usable slots, ~0.3 s match, ring LED) | 6-pin JST-SH 1.0 mm cable to the carrier PCB |
| RTC module | DS3231 (I2C, TCXO, battery-backed, +/-2 ppm) | 6-pin 2.54 mm header; shares I2C with the OLED |
| Display (optional) | SSD1306 0.96" 128x64 OLED, I2C address 0x3C | Taps the shared I2C bus; firmware runs headless if absent |
| Carrier PCB | Custom 2-layer interconnect board (KiCad, see 3.2) | No active parts; connectors + power/bus distribution |
| Enclosure | 3D-printed base + lid, plus an OLED bezel variant (see 3.3) | FDM, PLA/PETG |
| Power | 5 V USB supply into the carrier PCB's JST-GH 1.25 mm input | On-board 3V3 comes from the ESP32 module's regulator |

## 3.2 Custom carrier PCB

A 2-layer KiCad design (hardware/PCBs/Attendance System_v1/). It carries no active silicon and no regulator of its own — it is a wiring board that seats the ESP32 dev module and provides keyed, polarised connectors so the sensor, RTC, OLED, and power cannot be mis-plugged. It distributes the 5 V, 3V3, and GND rails and routes the UART (to the R503) and the shared I2C bus (RTC + OLED).

| Ref | Connector | Purpose |
| --- | --- | --- |
| J1 / J2 | 2 x 1x15, 2.54 mm pin socket | ESP32 dev-module headers |
| J3 | 1x6, JST-SH 1.0 mm vertical | R503 fingerprint sensor (VCC, TX, RX, GND, touch) |
| J4 | 1x6, 2.54 mm pin socket | DS3231 RTC + SSD1306 OLED — shared I2C (SDA/SCL), 3V3, GND; DS3231 32K / SQW broken out |
| J5 | 1x2, JST-GH 1.25 mm vertical | 5 V power input |

Nets of note: TX / RX (UART to the R503), SDA / SCL (I2C, shared by the DS3231 and OLED), 32K / SKW (DS3231 outputs, broken out but unused by the firmware). An earlier revision lives in hardware/PCBs/Attendance System_v0/.

## 3.3 3D-printed enclosure

Designed in FreeCAD (hardware/CAD files/Fingerprint Device Enclosure.FCStd) and exported to 3MF, sliced in Bambu Studio for an FDM printer. It is a two-part case:

- Base (Fingerprint Device Enclosure-Base.3mf) — Holds the carrier PCB and the ESP32 module; front face carries the R503 aperture and the power-cable entry.

- Lid (Fingerprint Device Enclosure-Lid.3mf) — Closes over the base (overall envelope on the order of 80 x 74 x 50 mm).

- OLED outer shell / bezel (Fingerprint Device Enclosure-OLED_Outer_Shell.3mf (~36 x 35 x 7 mm)) — Adds a framed window for the 0.96" display on display-equipped builds; the non-OLED build omits it. A combined variant is archived in Fingerprint Device Enclosure-OLED.zip.

Print in PLA or PETG. The sensor sits flush behind the front aperture; the lid clips onto the base. STLs/3MFs are print-ready; the FreeCAD file is the editable source.

## 3.4 Wiring

| Signal | ESP32 pin |
| --- | --- |
| R503 TX -> ESP32 RX | GPIO 16 |
| R503 RX -> ESP32 TX | GPIO 17 |
| DS3231 SDA | GPIO 21 |
| DS3231 SCL | GPIO 22 |
| OLED SDA | GPIO 21 (shared I2C bus with the DS3231) |
| OLED SCL | GPIO 22 (shared I2C bus with the DS3231) |

The R503 runs on a dedicated hardware UART (Serial2, 8N1). The DS3231 (0x68) and SSD1306 (0x3C) share one I2C bus brought up with Wire.begin(21, 22).

# 4. Firmware

A single Arduino sketch, firmware/ClassAttendance_Current_RTC/ClassAttendance_Current_RTC.ino. FIRMWARE_VERSION is 1.10.0. Three FreeRTOS tasks are pinned across the two cores.

## 4.1 Task model

| Task | Core | Responsibility |
| --- | --- | --- |
| FingerprintTask | 1 | Scan loop, match against the local fid map, LED feedback, enrollment execution, captive-portal launch on a master-finger scan. Writes each scan to SPIFFS, then signals NetworkTask via a binary semaphore. |
| NetworkTask | 0 | Provisioning, rotate-then-process SPIFFS queue flush to /log-attendance, NTP->RTC sync, WiFi reconnect, and a server-reachability watchdog (see 4.4). |
| EnrollmentTask | 0 | Polls /get-enrollment-job every 10 s, hands jobs to FingerprintTask, reports results durably (retried until acknowledged), and acts on the decommission signal. |

Inter-task: memQueueSem (binary semaphore) signals "a scan was persisted"; there is no in-RAM scan queue. Enrollment jobs pass through currentEnrollJob under enrollMutex. spiffsMutex serialises every SPIFFS access. Display state passes to the render path through a Core-0-safe mailbox — tasks never touch the display object directly.

## 4.2 Offline queue

Durability-first and crash-safe:

- Each scan is written to /queue.txt in SPIFFS synchronously, inside FingerprintTask, before any network attempt — never held only in RAM.

- On flush, NetworkTask atomically renames /queue.txt -> /queue_inflight.txt under the lock, then releases it, so FingerprintTask can keep appending to a fresh /queue.txt while the send runs.

- If power is cut mid-flush, recoverInflightQueue() merges the orphaned /queue_inflight.txt back into /queue.txt on the next boot, before tasks start.

- On send: 2xx drops the record; 5xx / 429 / network errors keep it for retry; a 4xx (server answered, payload rejected) drops it and does not feed the reachability watchdog.

- Caps: 1000 entries, 256 KB, 7-day age; oldest records are trimmed first (trim sweep every 5 min).

Each scan carries a scan_id of scan-YYYYMMDDhhmmss-<member-sid>, the timestamp read from the DS3231. scan_id is unique per institution and also deduplicated on (member, date, scan_type).

## 4.3 OLED display

If display.begin() succeeds, displayAvailable is set and the render path draws cards for: the idle clock + device name + online/offline, a scan verdict (name per the institution's member_name_display setting, plus Present / Time In / Time Out / cooldown / no-match), enrollment steps (PRESS 1/2, ENROLLED), the master-finger CONFIRM prompt, provisioning (PENDING + MAC), and OTA. A Tier-2 overlay on the idle screen shows the offline queue depth and dropped-record count. The panel sleeps after 120 s idle and wakes on the next event. If the panel is missing or fails to init, every one of these is a no-op.

## 4.4 Server-reachability watchdog

On a live WiFi link, postJSON*() feeds a health counter. After 5 consecutive failed POSTs the device is considered "server unreachable": it forces a WiFi re-associate every 60 s, and if it stays unreachable for 5 minutes it calls ESP.restart() to clear a wedged network stack. A 4xx response counts as reachable. An unrecoverable TLS-arena / low-heap condition also triggers a reboot.

## 4.5 SPIFFS layout

| File | Contents |
| --- | --- |
| /device_identity.json | device_id, institution_id, per-device device_secret, display_name (written after assignment) |
| /provisioning.json | device_id + provisioning_token during the pending window (deleted once assigned) |
| /fid_map.csv | fid,uniqueId,role,name — the slot -> member map used to resolve a scan |
| /queue.txt, /queue_inflight.txt | the durable attendance queue and its in-flight rotation |
| /scan_log.txt | rolling local scan log (size-capped) |
| device config cache | per-institution device-facing config (member_name_display, ...) with its config_rev |
| WiFi credentials | SSID / password captured by the captive portal |

# 5. Network & Security

## 5.1 TLS certificate validation

Every ESP32 HTTPS connection uses WiFiClientSecure with setCACert(ROOT_CA_BUNDLE). The bundle in certs.h must contain the full chain for each host — root AND every intermediate — because mbedTLS on the ESP32 cannot do AIA (Authority Information Access) fetching.

| Host | Chain in the bundle (fetched from live servers; refresh on CA rotation) |
| --- | --- |
| Supabase (*.supabase.co) | leaf -> WE1 (Google Trust Services) -> GlobalSign ECC Root CA R4 |
| GitHub API (api.github.com) | leaf -> Sectigo Public Server Auth CA DV E36 -> Sectigo Root E46 |
| GitHub release CDN (objects.githubusercontent.com) | leaf -> R12 (Let's Encrypt) -> ISRG Root X1 (ISRG Root X1 alone suffices) |

## 5.2 Authentication secrets

| Secret | Stored | Used for |
| --- | --- | --- |
| BOOTSTRAP_SECRET | secrets.h (gitignored, compiled in) / Supabase edge secret | x-bootstrap-secret on /register and /assignment-poll (pre-assignment only) |
| device_secret (per device) | SPIFFS /device_identity.json; devices.device_secret | x-device-secret on every operational call. A transitional path still accepts the legacy shared institutions.device_secret until all devices are re-provisioned. |
| provisioning_token | SPIFFS /provisioning.json; devices.provisioning_token | Binds /assignment-poll to the exact device that registered — prevents secret harvesting |
| CRON_SECRET | Supabase Vault | x-cron-secret on /mark-absent from pg_cron (the Authorization header is stripped by Kong, so a Bearer token cannot be used) |
| SUPABASE_SERVICE_ROLE_KEY | Vercel environment | The dashboard's server-side Supabase admin client |

## 5.3 Tenant isolation

The dashboard uses the Supabase service role, so RLS is bypassed. Isolation is enforced in application code: resolveInstitutionScope() locks every non-platform role to its own institution_id (query params are ignored), and every mutating server action calls ownsRecord(table, id, session) (lib/supabase/ownership.ts) before touching a row. platform_admin is cross-tenant by design. RLS policies ("service role full access") exist as dormant defence-in-depth. On the device side, /log-attendance resolves the member only within the authenticated device's institution, so a scan cannot cross tenants.

# 6. Database Schema

All tables are in public. institution_id is the tenant key on every scoped table (ON DELETE CASCADE). Migrations live in backend/supabase/migrations/ and are applied to cloud manually after review.

## 6.1 Core tables

| Table | Key columns | Notes |
| --- | --- | --- |
| institutions | id, name, type, timezone, label_* overrides, student_scan_mode, staff_scan_mode, tracked_weekdays, time_format, track_lateness, expected_start_time, late_grace_minutes, track_early_leaving, expected_end_time, early_leave_grace_minutes, member_name_display, currency, sell_products, sell_services, loyalty_enabled, status, device_secret, config_rev, theme_primary, theme_preset | One row per tenant. type: school ¦ office ¦ shop. tracked_weekdays (smallint[]) replaced the old skip_weekends flag. config_rev is bumped by the institutions_touch trigger on any change; the device polls it to refresh its cached config. |
| members | id, sid, fullname, group_name, unit_name, fin1, fin2, status, member_type, device_id, institution_id | member_type: student ¦ staff. sid is unique per institution. device_id FK ON DELETE SET NULL. |
| devices | id, group_name, unit_name, display_name, mac, provisioning_token, device_secret, revoked, institution_id | One ESP32 per (institution, group, unit). revoked=true -> instant 401. Devices enter only via physical provisioning. |
| periods | id, term, year, status, start_date, end_date, institution_id | Academic periods; nullable for office / shop tenants. |
| attendance | id, member_id, period_id, device_id, date, time, status, scan_type, punctuality, scan_id, institution_id | status: present ¦ absent. scan_type: present ¦ time_in ¦ time_out (CHECK). punctuality: on_time ¦ late ¦ early_leave ¦ null. date and time are in the institution timezone. device_id FK ON DELETE SET NULL. |
| enrollment_jobs | id, device_id, member_id, finger_slot, command, status, fid, note, allow_overwrite, dispatched_at, attempts, last_error, institution_id, created_at | command: register ¦ delete ¦ clearall ¦ register-master ¦ delete-master. member_id was renamed from student_id. allow_overwrite gates writing into an occupied sensor slot. dispatched_at, attempts and last_error drive re-delivery and auto-fail; a status guard makes completed / failed terminal. |
| holidays | id, label, start_date, end_date, recurring, institution_id | recurring=true: matched by month/day every year (year-wrap ranges supported). |
| profiles | id -> auth.users, role, assigned_unit, assigned_device_id, institution_id | Dashboard roles. assigned_device_id: required for teacher/staff (unresolved -> see nothing), optional for admin. platform_admin has null institution_id. |
| device_resets | device_id (uuid), created_at | Deferred SPIFFS-wipe queue. No FK — survives device-row deletion; not consumed on read. |
| institution_activity | institution_id (PK), last_change_at | Polling watermark maintained by triggers on attendance, members, devices and periods. Backs /api/changes; replaced the old SSE realtime stream. |

## 6.2 Retail / loyalty tables (shop tenants)

| Table | Purpose |
| --- | --- |
| clients | Identified buyers (name + phone, phone unique per tenant). Separate from members; no fingerprint. |
| products, services | Sellable catalog items (soft-deleted via active). products also carries stock. |
| client_attendance | One visit row per client per day (unique on institution_id, client_id, date). |
| transactions, transaction_items | A sale and its line items. Line items snapshot item_name / unit_price and carry their own institution_id. Written atomically by the create-sale function; discount_total / reward linkage supported. |
| rewards | Loyalty rule definitions — reward_kind + payload (product / service / value), condition_type, threshold, window, repeatable, auto. |
| rewards_log | Issuance + redemption records. transaction_id links a redeemed reward to the sale it was applied on; a guard prevents double-redeem and double-grant of non-repeatable rewards. |

# 7. Device Provisioning & Operation

## 7.1 First boot (new device)

- Boots with no /device_identity.json in SPIFFS. On the very first boot it also clears SPIFFS and restarts.

- Reads BOOTSTRAP_SECRET (compiled in). Connects to WiFi from saved credentials, or opens the captive portal "Attendance-Setup" at 192.168.4.1.

- Syncs time over NTP into the DS3231.

- POSTs { mac } to /register with the x-bootstrap-secret header.

- /register creates a devices row (unassigned), mints a provisioning_token, and returns { device_id, provisioning_token, status: "pending" }. The device saves both to /provisioning.json and shows a breathing-yellow LED.

- Polls /assignment-poll every 5 s with { device_id, provisioning_token }.

- An admin assigns the device to a group/unit on the dashboard /devices page.

- /assignment-poll returns { status: "assigned", institution_id, device_secret, display_name } — only if the provisioning_token matches.

- The device writes /device_identity.json, deletes /provisioning.json, and enters normal operation.

## 7.2 Normal operation

- Every scan: POST /log-attendance with the x-device-secret header (queued to SPIFFS first).

- Enrollment: poll POST /get-enrollment-job every 10 s; on a job, capture the finger and POST /update-enrollment-job (retried until acknowledged).

- OTA check on boot after a 0-120 s random jitter (see section 10).

- Config: /get-enrollment-job also returns member_name_display and config_rev; the device refreshes its cached device config when config_rev advances.

- Master finger: a scan of an enrolled master finger shows CONFIRM?; a second scan of the same finger within ~8 s reopens the WiFi captive portal.

## 7.3 Enrollment job lifecycle

A job is created pending on the dashboard. /get-enrollment-job hands it to the device and moves it to in_progress, stamping dispatched_at and incrementing attempts. The device enrols the finger and POSTs the result; /update-enrollment-job moves it to completed (with the FID) or failed (with last_error). If the result POST is lost (flaky TLS, a reboot between storeModel() and the POST), the job is re-delivered on a later poll; past a capped number of attempts it is auto-failed. The enrollment_jobs_status_guard trigger keeps a terminal status from being pulled back to in_progress by a late or raced write. Failed / stuck jobs expose Retry and Cancel on the dashboard.

## 7.4 Revocation & decommission

Set devices.revoked = true for an instant lockout — the device's next /log-attendance gets a 401. For a permanent wipe, insert a device_resets row: /get-enrollment-job then returns decommissioned: true on every poll (the row is not consumed on read, so it is durable against races) and the device wipes its identity and reboots into provisioning.

# 8. Edge Functions

Six Deno functions under backend/supabase/functions/; all are configured verify_jwt = false and do their own auth.

| Function | Caller | Auth | Purpose |
| --- | --- | --- | --- |
| /register | ESP32, first boot | x-bootstrap-secret | Create an unassigned devices row; issue a provisioning_token |
| /assignment-poll | ESP32, provisioning poll | x-bootstrap-secret + provisioning_token in body | Once an admin assigns the unit, return the per-device device_secret, institution_id, and display_name |
| /log-attendance | ESP32, every scan | x-device-secret (+ transitional shared-secret path) | Authenticate the device, resolve the member in-tenant, apply tracked-weekday / scan-mode / punctuality rules, dedupe, insert the row |
| /get-enrollment-job | ESP32 poll (10 s) | x-device-secret | Return the next job (+ member_name_display, config_rev); re-deliver stuck jobs; signal decommissioned |
| /update-enrollment-job | ESP32, after enrolling | x-device-secret | Record completed / failed and the FID; institution is derived from the device row, not the body |
| /mark-absent | pg_cron, daily (21:00 UTC) | x-cron-secret | Insert absent rows per institution/timezone; respects holidays, tracked weekdays, periods. Processes institutions in batches of 8; needs pg_net. |

# 9. Dashboard

Next.js App Router. This is a version with breaking API changes vs. stock Next.js — middleware lives in frontend/proxy.ts, not middleware.ts. Every Supabase call is server-side through the service role; no keys reach the browser.

## 9.1 Auth & role model

| Role | Scope | Access |
| --- | --- | --- |
| platform_admin | Cross-institution (null institution_id) | All pages; bypasses every role gate. Creates institutions and the first super_admin per tenant. |
| super_admin | One institution | All pages — devices, enrollment, promotion, academic, members, attendance, users, settings. |
| admin | One institution (optionally one device) | Members, academic, promotion, attendance, users (view + own password). No devices/enrollment. If assigned_device_id is set, records + enrollment are scoped to that device. |
| teacher / staff | One institution + one device | Read-only attendance and members, filtered to their device. An unresolved assignment shows an empty state, never everything. |
| cashier | One institution (shop) | Sales, Clients, Catalog only. No attendance, members, devices, or admin pages. |

dal.ts exports verifySession(), requireRole(...), getInstitution(), resolveInstitutionScope(), and resolveDeviceScope(). verifySession() is cache()-wrapped (one DB hit per render) and also gates suspended/deactivated tenants to /suspended. node scripts/check-rbac.mjs fails CI if any (dashboard) page lacks a requireRole( call.

## 9.2 Pages

| Route | Description |
| --- | --- |
| / | Overview — today's present/absent, attendance rate, recent scans. platform_admin sees a cross-tenant summary. |
| /attendance | Records with date / period / member / unit / status / type filters; per-member stats; Late / Early badges; CSV export. |
| /members, /staff | Rosters. /staff shows when track_staff = true. teacher/staff see their device only. |
| /devices | Device registry + provisioning: assign pending devices, set display name, revoke. super_admin / platform_admin. |
| /academic | Periods + holidays. Labelled "Academic" (school), "Periods & Holidays" (office), "Closed Days" (shop). |
| /enrollment | Enrollment job queue: register / delete / clearall / register-master / delete-master, with Retry / Cancel for failed or stuck jobs. |
| /promotion | Year-end bulk promotion. School tenants only. |
| /settings | Institution config: identity, brand colour, labels, per-type scan modes, tracked weekdays, punctuality thresholds, timezone, 12h/24h display, name-shown-on-device, and (shop) currency / offerings / loyalty. |
| /sales, /clients, /catalog, /rewards, /reports | Retail / loyalty module — shop tenants. /rewards and /reports are super_admin / admin; the rest also allow cashier. |
| /institutions, /institutions/[id], /onboarding | Tenant management — platform_admin only. |
| /users | Dashboard accounts. Email search + role filter. |
| /unauthorized, /suspended | Role-denied and tenant-suspended screens (outside the (dashboard) group). |

## 9.3 Installable PWA

A web manifest (app/manifest.ts -> /manifest.webmanifest, platform branding only) plus a minimal service worker (public/sw.js) make the dashboard installable to a home screen / desktop. The worker registers a passthrough fetch handler only — it never caches, so the auth-gated dashboard can't serve stale data — and is served no-cache. proxy.ts's matcher excludes /api and the PWA assets so a logged-out device can still read them. An in-app install prompt lives in components/pwa/.

# 10. OTA Firmware Updates

On boot, with WiFi up and no assignment pending, the device waits a random 0-120 s jitter (from hardware entropy) so a fleet-wide power blip doesn't hit GitHub's unauthenticated rate limit at once. It then GETs releases/latest from https://api.github.com/repos/sgariba21-beep/esp32-attendance-device/releases/latest.

Release tags must be firmware-v<major>.<minor>.<patch> (matching OTA_TAG_PREFIX). The tag version is compared against the compiled-in FIRMWARE_VERSION; only a strictly newer version flashes. The .bin asset is streamed through the ESP32 Update library (sensor LED holds red), and the device reboots into the new image on success. The API and CDN hosts must be in the TLS bundle (section 5.1).

# 11. Key File Locations

| Path | Purpose |
| --- | --- |
| firmware/ClassAttendance_Current_RTC/ClassAttendance_Current_RTC.ino | The firmware sketch |
| firmware/ClassAttendance_Current_RTC/certs.h | TLS root+intermediate bundle (ROOT_CA_BUNDLE) |
| firmware/ClassAttendance_Current_RTC/secrets.h | BOOTSTRAP_SECRET (gitignored; copy from secrets.example.h) |
| hardware/PCBs/Attendance System_v1/ | Custom carrier PCB (KiCad); v0 is the earlier revision |
| hardware/CAD files/Fingerprint Device Enclosure.FCStd | Enclosure source (FreeCAD); *.3mf are the print-ready parts |
| backend/supabase/functions/ | The six edge functions (Deno) |
| backend/supabase/migrations/ | Schema migrations — applied to cloud manually |
| backend/supabase/config.toml | Edge-function config (verify_jwt = false for all) |
| frontend/proxy.ts | Next.js middleware (NOT middleware.ts) |
| frontend/lib/supabase/dal.ts | verifySession / requireRole / resolveInstitutionScope / resolveDeviceScope |
| frontend/lib/supabase/ownership.ts | ownsRecord() — per-tenant ownership check |
| frontend/app/manifest.ts, frontend/public/sw.js | PWA manifest + service worker |
| docs/manuals-src/ | Client manual sources (build_manual.py -> docs/*.pdf) |

Generated from the repository at branch oled-integration. Regenerate this document when firmware contracts, the schema, or the edge-function API change.